APPENDIX 1

(A typical Specimen of Cover Page & Title Page)

INDUSTRIAL DOCTOR V2.0: An Edge-AI Based Real Time Predictive Maintenance System for Industrial Motors

A PROJECT REPORT

Submitted by

Shabaz Khan S

Mohamed Ishauq S

Abdul Razak

Mohamed Ashik M

in partial fulfilment for the award of the degree of

BACHELOR OF ENGINEERING

IN

ELECTRONICS AND COMMUNICATION ENGINEERING

AALIM MUHAMMED SALEGH COLLEGE OF ENGINEERING

ANNA UNIVERSITY: CHENNAI 600 025

APRIL  2026

APPENDIX 2

(A typical specimen of Bonafide Certificate)

ANNA UNIVERSITY: CHENNAI 600 025

BONAFIDE CERTIFICATE

Certified that this project report “…………………………………………….…”

is the Bonafide work of “……………………………………………….………”

who carried out the project work under my supervision.

| SIGNATURE Dr. A. S. Salma Banu HEAD OF THE DEPARTMENT, Department of Electronics and Communication Engineering Aalim Muhammed Salegh College, Avadi, Chennai – 600 054 | SIGNATURE Dr.K.G.Parthiban PROJECT SUPERVISOR, Professor Department of Electronics and Communication Engineering Aalim Muhammed Salegh College, Avadi, Chennai – 600 054 |
| --- | --- |

ABSTRACT

Industrial Doctor is a real-time predictive motor fault detection system designed for deployment on resource-constrained embedded hardware. The system addresses a critical gap in industrial maintenance practice: the inability to predict the first mechanical failure of a motor before it occurs, using only the data available from normal healthy operation. Conventional deep learning approaches to fault detection require thousands of labelled fault examples, GPU-class inference hardware, and full retraining whenever the motor model or operating environment changes. These requirements make such approaches fundamentally impractical for low-cost embedded deployment in real industrial settings.

The proposed system captures motor vibration and acoustic signals using a MEMS microphone and processes each 512-millisecond window through a six-feature extraction pipeline. The six features extracted are Root Mean Square (RMS), which measures effective signal energy; Kurtosis, which detects impulsive mechanical looseness; Crest Factor, which identifies sudden non-repetitive shocks; Peak Frequency, which identifies the motor's operating frequency fingerprint via Fast Fourier Transform; Spectral Centroid, which detects bearing friction through spectral brightness shifts; and Zero Crossing Rate (ZCR), which distinguishes rhythmic motor signals from chaotic ambient noise.

Each extracted feature is normalised using the Z-Score formula Z = (x − μ) / σ, where μ and σ represent the mean and standard deviation computed from clean healthy motor recordings at 6V and 8V operating voltages. This statistical normalisation converts all six features — regardless of their original units or scale — into a universal deviation measure, enabling direct comparison and weighted combination. A weighted voting engine aggregates the six Z-Scores in real time, assigning sensitivity-based weights: Kurtosis receives a weight of 2.0, Spectral Centroid 1.8, Crest Factor 1.5, and the remaining features 1.0 each. An alert is fired when the normalised weighted fault score exceeds a threshold of ±2.5 across three consecutive windows, a condition that statistically corresponds to readings that only 0.3% of healthy motor operation would produce.

The complete inference pipeline runs on an ESP32 microcontroller using only integer arithmetic, achieving sub-millisecond decision latency with no external memory, a hardware single-precision Floating-Point Unit (FPU), and no network connectivity required. The system successfully distinguishes four operational states — healthy motor, mechanical fault, ambient crowd noise, and motor-off silence — and provides full explainability for every alert by reporting which features voted highest and the probable fault type. The Z-Score baseline can be recalibrated for a new motor in minutes by collecting fresh healthy recordings, requiring no retraining of any kind.

APPENDIX 3

(A typical specimen of table of contents)

TABLE OF CONTENTS

| CHAPTER NO. 1. 2. 3. 4. 5. 6. | TITLE ABSTRACT LIST OF TABLES LIST OF FIGURES LIST OF SYMBOLS, ABBREVIATIONS AND NOMENCLATURE INTRODUCTION - BACKGROUND AND MOTIVATION - PROBLEM STATEMENT - OBJECTIVES OF THE WORK - SCOPE AND CONSTRAINTS - ORGANIZATION OF THE REPORT LITERATURE REVIEW - GENERAL - TRADITIONAL MOTOR FAULT DETECTION METHODS - DEEP LEARNING APPROACHES TO FAULT DETECTION - STATISTICAL ANOMALY DETECTION IN INDUSTRIAL SYSTEMS - MEMS-BASED VIBRATION SENSING - GAPS IDENTIFIED IN PRIOR WORK SIGNAL PROCESSING AND FEATURE EXTRACTION 3.1 GENERAL 3.2 SIGNAL ACQUISITION — MEMS MICROPHONE 3.3 FEATURE 01: ROOT MEAN SQUARE 3.4 FEATURE 02: KURTOSIS 3.5 FEATURE 03: CREST FACTOR 3.6 FEATURE 04: PEAK FREQUENCY 3.7 FEATURE 05: SPECTRAL CENTROID 3.8 FEATURE 06: ZERO CROSSING RATE (ZCR) 3.9 FEATURE-STATE BEHAVIOUR MATRIX - SUMMARY Z-SCORE STATISTICAL INTELLIGENCE ENGINE - GENERAL - WHAT IS Z-SCORE - THE FORMULA UNPACKED - Z-SCORE IN SOUND SIGNAL ANALYSIS - Dynamic Thresholding - Ambient Noise Veto - Sensor Fusion - Trend Detection - Outlier Immunity - Feature Normalisation - STANDARD AI VS Z-SCORE STRATEGY - THE Z-SCORE VOTING SYSTEM - Baseline Collection - Per-Window Scoring - Weighted Vote Computation - Threshold Decision - DATASET STRATEGY - Dead Silent Session — Floor Baseline - Healthy 6V / 8V — Operating Baselines - Healthy Varied — Robustness Baseline - Crowd / Ambient — Veto Calibration - SUMMARY RESULTS AND DISCUSSION - GENERAL - FEATURE BEHAVIOUR ACROSS MOTOR STATES - Z-SCORE VOTING PERFORMANCE - FALSE POSITIVE SUPPRESSION - SYSTEM LATENCY AND RESOURCE UTILISATION - COMPARISON WITH NEURAL NETWORK BASELINE - DISCUSSION CONCLUSION AND FUTURE WORK - SUMMARY OF WORK - LIMITATIONS - FUTURE WORK | PAGE NO 1 1 2 3 4 6 9 9 9 10 11 12 13 14 15 16 17 15 17 18 19 20 21 20 23 24 25 26 26 27 27 28 28 28 28 29 29 32 32 32 32 33 35 37 37 37 38 38 40 40 40 41 42 43 44 45 47 47 48 48 |
| --- | --- | --- |

LIST OF TABLES

| TABLE NO. | TITLE | PAGE NO. |
| --- | --- | --- |
| Table 3.1 | Feature-State Behaviour Matrix — All Six Features Across Four Motor States | 22 |
| Table 4.1 | Z-Score Zone Reference — Threshold Levels and Corresponding System Actions | 25 |
| Table 4.2 | Standard AI vs. Z-Score Voting — Comparative Criteria | 30 |
| Table 4.3 | Z-Score Voting System — Weighted Feature Scores and Decision Logic | 33 |
| Table 4.4 | Dataset Strategy — Recording Sessions and Their Role in the Z-Score Engine | 35 |
| Table 5.1 | Classification Results — Z-Score Voting System Across Four Motor States | 41 |
| Table 5.2 | System Latency and Resource Utilisation on ESP32 Microcontroller | 43 |
| Table 5.3 | Hardware Comparison — Industrial Doctor vs. Neural Network Baseline | 44 |

LIST OF FIGURES

| FIGURE NO. | TITLE | PAGE NO. |
| --- | --- | --- |
| Figure 3.1 |  | 15 |
| Figure 3.2 | Kurtosis — Healthy Signal (K = 3) vs. Mechanical Fault (K > 10) | 16 |
| Figure 3.3 |  | 17 |
| Figure 3.4 | Peak Frequency — FFT Magnitude Spectrum with Fundamental Peak and Harmonic Family | 19 |
| Figure 3.5 | Spectral Centroid — Healthy Spectrum vs. Fault Spectrum Showing Centroid Shift | 20 |
| Figure 3.6 | Zero Crossing Rate — Motor Signal (Low ZCR) vs. Ambient Noise (High ZCR) | 21 |
| Figure 4.1 | Bell Curve (Normal Distribution) — Z-Score Zones from -3 to +3 with Fault Detection Threshold | 26 |
| Figure 4.2 | Real Data Strip — Healthy 8V Baseline with Fault Reading at Z = +2.0 | 27 |
| Figure 4.3 |  | 29 |
| Figure 4.4 | Conceptual Comparison — Neural Network vs. Z-Score Voting System Architecture | 31 |
| Figure 4.5 | Voting Bar Chart — Each Feature Casts a Weighted Z-Score Vote with Alert Threshold | 35 |
| Figure 4.6 | Data Pipeline — Recording Sessions to Z-Score Baselines to Live Fault Detection | 38 |

LIST OF SYMBOLS, ABBREVIATIONS AND NOMENCLATURE

| SYMBOL / ABBREVIATION | MEANING / EXPANSION |
| --- | --- |
| SECTION A: MATHEMATICAL SYMBOLS AND NOTATION |  |
| Z | Z-Score — standardised deviation from the healthy mean |
| x | Current sensor reading (raw feature value) |
| μ (mu) | Mean of healthy baseline data |
| σ (sigma) | Standard deviation of healthy baseline data |
| N | Window length (number of samples in one analysis frame) |
| xi | i-th acceleration or audio sample in a window |
|  | RMS: Root Mean Square |
| K | Kurtosis — statistical peakiness of amplitude distribution |
| CF |  |
| f* | Peak Frequency — dominant frequency from FFT |
| SC | Spectral Centroid — centre of mass of frequency spectrum |
| ZCR | Zero Crossing Rate — number of sign changes per unit time |
| FFT | Fast Fourier Transform |
| sgn(x) | Sign function — returns +1 or -1 based on polarity of x |
| ±2.5 | Alert threshold for the weighted Z-Score voting system |
| ±3 | Fault alert boundary (only 0.3% of healthy readings reach this) |
| ms | Millisecond — unit of time for inference latency |
| Hz | Hertz — unit of frequency |
| V | Volt — unit of motor supply voltage |
| SECTION B: ABBREVIATIONS |  |
| AI | Artificial Intelligence |
| BNN | Binary Neural Network |
| CPU | Central Processing Unit |
| DSP | Digital Signal Processing |
| IoT | Internet of Things |
| MEMS | Micro-Electro-Mechanical Systems |
| ML | Machine Learning |
| PCB | Printed Circuit Board |
| RAM | Random Access Memory |
|  | RMS: Root Mean Square |
| SNR | Signal-to-Noise Ratio |
| ESP32 | ESP32 S3 microcontroller |
|  | Universal Asynchronous Receiver-Transmitter |
|  | Universal Serial Bus |
| ZCR | Zero Crossing Rate |
| FSM | Finite State Machine |
| GPIO | General-Purpose Input/Output |
| GPU | Graphics Processing Unit |
| HDL | Hardware Description Language |
| SoC | System on Chip |
| SPI | Serial Peripheral Interface |
| SECTION C: NOMENCLATURE |  |
| 512ms | Duration of one analysis window — the time frame over which all six features are computed |
| 6V / 8V | Motor supply voltage operating levels used for baseline calibration |
| 40 – 55 Hz | Typical Peak Frequency range of the motor operating at 6V supply |
| 60 – 75 Hz | Typical Peak Frequency range of the motor operating at 8V supply |
| Z = 0 | Perfect normal — current reading equals the healthy baseline mean exactly |
| Z = ±2 | Suspicious zone — reading is slightly outside normal; log and monitor for trend |
| Z = ±2.5 | Decision threshold — weighted fault score must exceed this for three consecutive windows to trigger an alert |
| Z = ±3 | Fault alert boundary — only 0.3% of healthy readings statistically reach this level |
| Z > ±4 | Critical zone — statistically near-impossible in healthy operation; immediate stop recommended |
| 0.20 – 0.80 |  |
| 2.5 – 3.5 | Kurtosis range of a healthy motor — near-Gaussian amplitude distribution |
| 1.4 – 2.5 | Crest Factor range of a healthy motor at constant sinusoidal load |
| > 10 | Kurtosis threshold indicating repetitive mechanical impulses — looseness or bearing fault |
| > 6.0 | Crest Factor threshold indicating early bearing spalling impulses |
| < 0.05 |  |
| Kurtosis × 2.0 | Weight assigned to Kurtosis in the voting system — highest due to bearing fault sensitivity |
| SC × 1.8 | Weight assigned to Spectral Centroid — second highest due to early friction detection |
| CF × 1.5 | Weight assigned to Crest Factor — third highest due to shock sensitivity |
|  | Base weight assigned to remaining three features in the voting system |
| 3 consecutive windows | Persistence requirement for alert firing — prevents false alarms from isolated spikes |
| < 9 ms | Total inference latency of the complete feature extraction and voting pipeline on ESP32 |
| < 96 bytes | Flash memory required to store all 12 baseline mu-sigma pairs |
| < 2 KB | RAM usage of the complete Industrial Doctor V2.0 inference pipeline |
| Healthy_6V | Dataset label for clean motor recordings at 6V — primary operating baseline |
| Healthy Varied | Dataset label for recordings during real-world load variation — robustness baseline |
| Dead Silent | Dataset label for recordings with motor off — ambient noise floor baseline |
| Crowd Ambient | Dataset label for environmental noise recordings — veto calibration baseline |
| Industrial Doctor Alert | System-level fault event fired when weighted Z-Score vote exceeds threshold for 3 windows |

CHAPTER 1

INTRODUCTION

  - BACKGROUND AND MOTIVATION

The rapid expansion of industrial automation and smart manufacturing has created an urgent demand for reliable, low-cost systems that can monitor the health of electric motors in real time. Electric motors are among the most widely deployed machines in industrial facilities worldwide, powering pumps, fans, conveyors, compressors, and robotic actuators. When a motor fails unexpectedly, the consequences extend well beyond the cost of the replacement component: unplanned downtime in manufacturing environments can cost thousands of dollars per hour, and in safety-critical installations such as water treatment plants, HVAC systems, and medical equipment, an unexpected motor failure may pose direct risks to personnel and operational continuity.

Conventional maintenance strategies fall into two broad categories. Reactive maintenance — replacing components only after they have failed — minimises upfront expenditure but accepts the full cost of unplanned downtime and emergency repairs. Preventive maintenance — replacing components on fixed time schedules regardless of their actual condition — reduces failure risk but introduces unnecessary replacement costs and wastes the remaining useful life of healthy components. Both strategies share a fundamental limitation: they do not use the motor's own operational data to inform maintenance decisions.

Predictive maintenance addresses this limitation by continuously monitoring the physical condition of a machine and intervening only when     measurable indicators suggest that a failure is imminent. The challenge lies in acquiring relevant condition data, extracting meaningful features from that data, and making reliable fault decisions in real time — ideally on low-cost hardware that can be deployed at scale across an industrial facility without requiring dedicated servers or cloud connectivity.

Industrial Doctor V2.0 is designed to meet precisely this challenge. By capturing the acoustic and vibration signature of a running motor using a MEMS microphone, extracting six carefully chosen signal features from each analysis window, and applying a statistical Z-Score anomaly detection engine, the system detects the onset of mechanical faults before they become catastrophic failures — using only the knowledge of what the motor sounds like when it is healthy.

1.2 PROBLEM STATEMENT

The central problem this project addresses can be stated as follows: given the hardware profile of a low-cost ESP32-S3 microcontroller and a MEMS microphone, it is not feasible to deploy a standard deep learning fault classifier for real-time motor condition monitoring. Three specific constraints define the problem boundary.

First, conventional neural network-based fault classifiers require large quantities of labelled fault data for training. A motor that has not yet experienced a failure — which is precisely the situation in a new predictive maintenance deployment — cannot provide fault examples. The system must therefore be capable of detecting anomalies using only healthy baseline data.

Second, embedded microcontrollers such as the ESP32 series our system can run on integer-only hardware but leverages the ESP32-S3's hardware FPU for even higher performance and memory bandwidth required to execute neural network inference at the sample rates demanded by real-time motor monitoring. The inference algorithm must be reducible to simple integer arithmetic executable within the microcontroller's computational budget.

Third, the deployed system must be robust to environmental noise — crowd noise, HVAC hum, nearby machinery — that shares frequency and amplitude characteristics with motor fault signals. A naive threshold on any single feature would produce unacceptable false positive rates in a realistic industrial environment. The detection strategy must fuse multiple complementary features to distinguish genuine motor faults from environmental interference.

1.3 OBJECTIVES OF THE WORK

The primary objectives of this project are as follows:

1.  To design and implement a six-feature signal extraction pipeline — comprising RMS, Kurtosis, Crest Factor, Peak Frequency, Spectral Centroid, and Zero Crossing Rate — capable of capturing the full mechanical and spectral behaviour of a running electric motor from a single MEMS microphone input.

2.  To establish healthy baseline profiles for each extracted feature at 6V and 8V operating voltages by computing the statistical mean and standard deviation from clean motor recordings, and to store these baselines in the microcontroller's flash memory for runtime reference.

3.  To implement a Z-Score normalisation engine that converts each raw feature reading into a universal deviation score, enabling direct comparison and combination of features with fundamentally different units and scales.

4.  To design a weighted Z-Score voting system that aggregates the six per-feature anomaly scores into a single normalised fault index, with weights assigned according to each feature's sensitivity to specific fault types, and to fire an alert when the fault index exceeds a statistically justified threshold across three consecutive analysis windows.

5.  To validate the complete system across four distinct motor operational states — healthy operation, mechanical fault, ambient crowd noise, and motor-off silence — and to demonstrate that the voting system correctly classifies each state with minimal false positives and missed detections.

6.  To deploy the complete inference pipeline on an ESP32 microcontroller and to confirm that all six feature extractions, Z-Score computations, and voting decisions are completed within the 512-millisecond analysis window using only on-chip arithmetic resources.

1.4 SCOPE AND CONSTRAINTS

The scope of this project is strictly defined by the synergy between the selected hardware ecosystem and the statistical datasets cultivated during the implementation phase. To maintain a focus on cost-effective, deployable industrial solutions, the project adheres to the following boundaries:

Hardware and Edge-AI Integration

The computational substrate for this system is the ESP32-S3 microcontroller, selected for its integrated AI acceleration instructions and dual-core efficiency. It is paired with a high-sensitivity MEMS microphone for non-contact acoustic signal acquisition.

- Edge Autonomy: A critical constraint of this work is the "all-on-chip" requirement. All signal processing, feature extraction (Time and Frequency domains), and the Z-Score voting logic are executed locally on the ESP32-S3.

- Infrastructure Independence: The system operates without reliance on external servers, cloud-based APIs, or expensive dedicated Digital Signal Processing (DSP) hardware, ensuring functionality in air-gapped or remote industrial environments.

Operational Parameters and Scalability

The system is optimized for single-motor monitoring within specific power profiles.

- Voltage Contexts: The current logic is trained for two primary operating states: 6V and 8V. The system uses the identified Peak Frequency to automatically toggle between these two baseline profiles.

- Environmental Specificity: Performance metrics reported herein are characteristic of the specific motor-microphone-environment "acoustic signature" captured during the training phase.

- Rapid Recalibration: Unlike traditional AI models that require extensive retraining, the scope allows for rapid field adaptation. If deployed on a different motor model, the system only requires a few minutes of "healthy" data collection to recompute the Z-Score baselines ($\mu$ and $\sigma$), requiring zero changes to the source code.

Sensing Modality and Diagnostic Depth

The project prioritizes non-invasive monitoring over internal diagnostics.

- Acoustic vs. Kinetic: The system relies entirely on acoustic emissions captured through the air. It deliberately excludes mechanical motor disassembly or contact-based accelerometer measurements. This simplifies installation but introduces the challenge of ambient noise interference, which the ZCR-based veto logic is designed to mitigate.

- Anomaly vs. Localization: The system is designed as an Anomaly Detection Engine rather than a precise failure localization tool. While it can suggest "Probable Fault Categories" (e.g., bearing friction or mechanical looseness) based on feature-fault correlations, it does not claim to pinpoint specific subcomponent failures (like a specific cracked ball in a bearing) with absolute certainty.

1.5 ORGANIZATION OF REPORT

The remainder of this report is systematically organized into five distinct chapters, each designed to transition the reader from theoretical foundations to the practical implementation and empirical validation of the system.

Chapter 2: Literature Review

This chapter provides a comprehensive survey of the current state of the art in predictive maintenance. It begins with an analysis of traditional motor fault detection methods, such as Motor Current Signature Analysis (MCSA) and contact-based vibration analysis. It then evaluates the rise of Deep Learning approaches, discussing their high accuracy alongside the "cold-start" data requirements that limit their industrial utility. Finally, the chapter explores emerging literature on statistical anomaly detection, providing a theoretical justification for why Z-Score-based Edge-AI is a superior fit for resource-constrained embedded environments.

Chapter 3: Signal Processing and Feature Extraction

This chapter serves as the technical blueprint for the "Industrial Doctor" sensing pipeline. It offers a granular breakdown of the six distinct features extracted from the acoustic signal: RMS, Kurtosis, Crest Factor, Peak Frequency, Spectral Centroid, and Zero Crossing Rate (ZCR). Each feature is analyzed through three lenses:

- The mathematical derivation optimized for integer arithmetic.

- The physical interpretation (e.g., how Kurtosis maps to mechanical impacts).

- The predictive behavior, detailing how these values shift across the four operational states (Healthy, Fault, Ambient, and Silent).

Chapter 4: Z-Score Statistical Intelligence Engine

This chapter details the core analytical framework of the project. It introduces the Z-Score concept as a universal language for sensor fusion and provides a step-by-step formula derivation. Significant focus is placed on the Weighted Voting Architecture, explaining the logic behind assigning sensitivity weights to specific features. Additionally, it outlines the Dataset Strategy, describing the methodology for capturing baseline healthy data and the "Ambient Veto" calibration used to suppress false positives.

Chapter 5: Results and Discussion

In this chapter, the performance of the implemented system is critically evaluated using real-world datasets. It presents classification performance metrics across all motor states, emphasizing the system’s False Positive Suppression capabilities in high-noise environments. Technical benchmarks, including inference latency and CPU/RAM utilization on the ESP32-S3, are reported. The chapter concludes with a comparative analysis, pitting the Z-Score Voting System against a standard Neural Network baseline to highlight the advantages in speed, explainability, and data efficiency.

Chapter 6: Conclusion and Future Work

The final chapter synthesizes the findings of the project, summarizing the primary contributions to the field of Edge-AI maintenance. It provides a candid assessment of the identified limitations, such as the challenges of non-contact sensing in extreme acoustic environments. Finally, it proposes future research directions, including the integration of multi-modal sensing and the development of continuous voltage-interpolation baselines.

CHAPTER 2

LITERATURE REVIEW

2.1 GENERAL

The detection of mechanical faults in electric motors has been an active area of research for several decades, with approaches spanning classical signal processing, expert systems, and, more recently, deep learning. This chapter surveys the body of work that most directly informs the design decisions of Industrial Doctor V2.0, organised across four thematic areas: traditional vibration and acoustic analysis methods, deep learning-based fault classifiers, statistical anomaly detection frameworks, and MEMS-based sensing for embedded industrial systems. The chapter concludes by identifying the specific gaps in prior art that the present project addresses.

2.2 TRADITIONAL MOTOR FAULT DETECTION METHODS

The earliest systematic approaches to motor fault detection relied on direct physical measurements — vibration accelerometers, current signature analysis, and thermal imaging — processed through hand-crafted feature extractors and threshold-based classifiers. Motor Current Signature Analysis (MCSA), introduced in the 1980s, identifies fault-related frequency components in the stator current spectrum that arise from air-gap eccentricity, broken rotor bars, and bearing defects. MCSA requires access to the motor's electrical supply connections and specialist signal conditioning hardware, making it unsuitable for retrofitting to motors already installed in the field without electrical modification.

Vibration analysis using piezoelectric accelerometers has been the dominant method for bearing fault detection in rotating machinery. The characteristic defect frequencies of a bearing — the ball pass frequency on the outer race (BPFO) and inner race (BPFI) — can be computed from the bearing geometry and shaft speed, and their presence in the vibration spectrum provides highly specific fault identification. However, contact accelerometer methods require physical mounting on the motor housing, careful attention to sensor coupling, and post-processing by trained engineers who can interpret the resulting spectra. These requirements limit deployability in low-cost, high-volume monitoring scenarios.

Acoustic emission sensing provides a non-contact alternative to accelerometers and has been demonstrated to detect early-stage bearing spalling before vibration-based methods register anomalies. The limitation of acoustic emission in industrial environments is its extreme sensitivity to ambient noise: even moderate background noise levels can mask the high-frequency signals associated with early bearing damage, requiring sophisticated noise cancellation strategies that add computational and hardware cost.

2.3 DEEP LEARNING APPROACHES TO FAULT DETECTION

The success of deep learning in image and speech recognition has motivated extensive research into its application for motor fault detection. Convolutional Neural Networks (CNNs) applied directly to raw vibration waveforms or their Short-Time Fourier Transform (STFT) spectrograms have demonstrated accuracy figures exceeding 98% on standard benchmark datasets such as the Case Western Reserve University (CWRU) bearing dataset. Recurrent architectures including Long Short-Term Memory (LSTM) networks have been applied to temporal sequences of vibration features, exploiting the time-series structure of fault progression to achieve high sensitivity to gradual degradation.

Despite these impressive benchmark results, deep learning approaches share several structural limitations that make them unsuitable for the deployment scenario targeted in this project. Training a deep learning fault classifier requires a balanced dataset of both healthy and fault-condition examples. In a new industrial deployment — where the motor has not yet experienced a failure — fault examples simply do not exist. Collecting them by deliberately damaging a motor is impractical in most operational contexts. This cold-start problem is the fundamental barrier to deep learning deployment in predictive maintenance applications where early fault detection is most valuable.

Furthermore, neural network inference on embedded microcontrollers requires either model compression techniques such as quantization and pruning, or dedicated neural processing unit (NPU) hardware. While microcontrollers with integrated NPUs are commercially available, they carry a significant cost premium over general-purpose ARM Cortex-M devices and introduce supply chain dependencies that reduce deployability at scale.

2.4 STATISTICAL ANOMALY DETECTION IN INDUSTRIAL SYSTEMS

Statistical anomaly detection approaches sidestep the cold-start problem by modelling only the normal operating condition and flagging deviations as anomalous. Principal Component Analysis (PCA)-based monitoring has been widely applied in process industries to detect shifts in multivariate sensor readings relative to a healthy operating envelope. Control chart methods such as Shewhart charts and CUSUM (Cumulative Sum) control charts have been applied to individual process variables for decades and provide well-understood statistical guarantees on false alarm rates.

The Z-Score, as a single-variable standardisation technique, is conceptually related to these multivariate methods but offers a critical advantage for embedded deployment: its computation requires only the mean and standard deviation of the baseline distribution, both of which can be stored as scalar values in microcontroller flash memory, and the per-sample computation requires only subtraction and division — operations executable in a single clock cycle on integer hardware. The extension of per-feature Z-Scores to a weighted voting architecture, as implemented in this project, provides a multi-feature fusion capability comparable in power to PCA-based monitoring while remaining entirely tractable on an ESP32-S3 microcontroller.

2.5 MEMS-BASED VIBRATION SENSING

The commercialisation of MEMS microphones and accelerometers over the past decade has dramatically reduced the cost and size of vibration-sensing hardware. MEMS microphones capable of capturing acoustic frequencies up to 20 kHz are available for under one US dollar, making them viable candidates for large-scale motor monitoring deployments where per-unit sensor cost is a primary constraint. Several research groups have demonstrated MEMS microphone-based motor monitoring systems, establishing that the acoustic signature captured by a non-contact MEMS microphone placed in proximity to a running motor contains sufficient information for state classification.

The primary limitation of MEMS microphone-based monitoring, relative to contact accelerometers, is its sensitivity to ambient acoustic noise. A MEMS microphone captures all acoustic energy reaching it, including environmental noise sources that may be unrelated to the motor under test. This limitation motivates the use of multi-feature fusion strategies — such as the Z-Score voting system implemented in this project — that can identify and suppress environmental noise signatures rather than passing them through as false fault alerts.

2.6 GAPS IDENTIFIED IN PRIOR WORK

A review of the literature surveyed in this chapter reveals three consistent gaps between the existing body of work and the requirements of the present project. The first gap is the cold-start problem: virtually all deep learning fault classifiers require labelled fault data for training, which is unavailable in a new predictive maintenance deployment. The second gap is hardware cost and complexity: most demonstrated systems assume the availability of dedicated signal processing hardware, contact accelerometers, or cloud connectivity, none of which are present in the target deployment scenario. The third gap is noise robustness: published MEMS microphone-based monitoring systems that operate in genuinely noisy industrial environments are rare, and those that exist typically employ deep learning noise suppression models that reintroduce the hardware cost and cold-start problems.

Industrial Doctor V2.0 directly addresses all three gaps. By operating exclusively on healthy baseline data, deploying entirely on a low-cost ESP32-S3 microcontroller, and using a multi-feature Z-Score voting architecture specifically designed to suppress ambient noise false positives, the system fills the design space that prior work has left unaddressed.

CHAPTER 3

SIGNAL PROCESSING AND FEATURE EXTRACTION

3.1 GENERAL

This chapter presents the complete six-feature signal extraction pipeline that forms the input to the Z-Score anomaly detection engine. Each feature is described in terms of its mathematical definition, its physical interpretation in the context of motor vibration acoustics, and its characteristic behaviour across the four operational states that the system is designed to classify: healthy motor operation, mechanical fault, ambient crowd noise, and motor-off silence. The chapter concludes with a unified feature-state behaviour matrix that summarises the discriminative profile of all six features simultaneously.

3.2 SIGNAL ACQUISITION — MEMS MICROPHONE

The system acquires motor acoustic and vibration signals using a MEMS microphone mounted in proximity to the motor under test. The microphone outputs a time-series of amplitude samples representing the acoustic pressure of 512 milliseconds duration. Each window contains N samples, where N is determined by the microphone's sampling rate. Feature extraction is performed independently on each window, and the resulting six-feature vector is passed to the Z-Score engine for anomaly scoring. This windowed approach provides a balance between temporal resolution — sufficient to detect rapidly developing faults — and statistical stability of the extracted features.

3.3   FEATURE 01: ROOT MEAN SQUARE (RMS)

Root Mean Square is defined as the square root of the arithmetic mean of the squared sample values within a window. For a window of N samples x1, x2, ..., xN, the RMS is computed as:

RMS = sqrt( (1/N) * sum(xi^2) )

RMS measures the effective power or energy of the vibration signal across the analysis window. It serves as the primary load indicator in the system: a near-zero RMS confirms that the motor is switched off, while a sustained elevated RMS confirms mechanical activity. A healthy motor operating under normal load produces a stable RMS in the range 0.20 to 0.80. A mechanical fault involving overload or severe vibration surge drives RMS above 1.20. The limitation of RMS as a standalone fault indicator is its susceptibility to ambient noise: crowd noise and environmental machinery can elevate RMS into the healthy motor range without any motor activity, which is why the remaining five features are essential for correct state classification.

RMS Signal — Motor Running Zone vs. Idle Zone (Time Domain), as illustrated in Figure 3.1

3.4 FEATURE 02: KURTOSIS

Kurtosis is the fourth standardised central moment of the amplitude distribution within a window, defined as:

K = [ (1/N) * sum((xi - mu)^4) ] / sigma^4

where mu is the sample mean and sigma is the sample standard deviation of the window. Kurtosis measures the statistical peakiness of the amplitude distribution. A purely Gaussian signal has a theoretical Kurtosis of 3.0. A healthy motor operating under stable rotational load produces a near-Gaussian vibration distribution, with Kurtosis values in the range 2.5 to 3.5. Repetitive mechanical impacts — a loose screw rattling against its mounting, a cracked or spalled bearing surface making periodic contact — introduce sharp, periodic spikes into the vibration signal that are not Gaussian, driving Kurtosis above 10.

The temporal variance of Kurtosis is as important as its instantaneous value. A persistently elevated Kurtosis with low variance over successive windows indicates a structural, periodic fault source — the signature of mechanical looseness. A Kurtosis that fluctuates widely between windows indicates chaotic, non-structural energy — the signature of ambient noise. This variance-based disambiguation is incorporated into the voting system's fault classification logic.

Kurtosis — Healthy Signal (K = 3) vs. Mechanical Fault (K > 10), as illustrated in Figure 3.2

3.5 FEATURE 03: CREST FACTOR

Crest Factor is defined as the ratio of the peak absolute amplitude to the RMS of the window:

CF = max|x| / RMS(x)

For a pure sinusoidal signal, the theoretical minimum Crest Factor is the square root of 2, approximately 1.414. A healthy motor operating under constant sinusoidal load produces a stable Crest Factor near this theoretical minimum, in the range 1.4 to 2.5. Crest Factor is sensitive to sudden, non-repetitive shocks that Kurtosis may smooth over in averaged windows. A single large mechanical impulse — a bearing ball striking a spall pit, a loose rotor component making brief contact with the stator — keeps the window RMS low while driving the peak amplitude high, producing a very large Crest Factor. Early bearing spalling therefore elevates Crest Factor significantly before the RMS increases, making it one of the earliest available indicators of incipient mechanical faults.

Crest Factor — Low RMS Baseline with High-Amplitude Shock Pulse, as illustrated in FIGURE 3.3

\\3.6

3.6 FEATURE 04: PEAK FREQUENCY

Peak Frequency is the dominant frequency component identified in the Fast Fourier Transform (FFT) magnitude spectrum of the analysis window:

f* = argmax_f |X(f)|

where X(f) is the FFT output. Each supply voltage produces a distinct fundamental rotational frequency — at 6V the motor operates near 40 to 55 Hz, and at 8V it shifts to approximately 60 to 75 Hz. The Peak Frequency feature serves two functions in the system. First, it acts as a context switcher: by identifying the dominant frequency, the inference engine determines which voltage-mode baseline to use for subsequent feature comparisons. Second, it provides a direct diagnostic signal for fault frequency modulation: mechanical faults introduce sideband frequencies flanking the fundamental, which shift the identified peak frequency away from its healthy value.

Ambient noise produces no narrow dominant spectral peak — it spreads acoustic energy broadly and uniformly across the spectrum, leaving the Peak Frequency undefined or variable. This characteristic makes Peak Frequency a powerful discriminator between motor-related signals and broadband environmental noise.

Peak Frequency — FFT Magnitude Spectrum with Fundamental Peak and Harmonic Family, as illustrated in Figure 3.4

3.7 FEATURE 05: SPECTRAL CENTROID

Spectral Centroid is the centre of mass of the frequency spectrum, computed as the magnitude-weighted average of all frequency bins:

SC = sum(fk * |X(fk)|) / sum(|X(fk)|)

Spectral Centroid represents the perceived spectral brightness of the acoustic signal. A healthy motor concentrates its energy near the fundamental frequency and its lower harmonics, producing a low Spectral Centroid. As bearing surfaces degrade through friction, microscopic asperities generate a broadband high-frequency hissing sound that shifts the spectral centre of mass upward — the spectrum becomes perceptibly brighter. This shift occurs long before the motor physically seizes or before Kurtosis crosses its fault threshold, making Spectral Centroid the earliest available indicator of bearing friction and degradation.

Human speech and crowd noise produce very high Spectral Centroids in the range of 1 to 4 kHz, far above the typical motor fundamental frequency range. This makes Spectral Centroid a useful secondary indicator for ambient noise detection when combined with ZCR.

Spectral Centroid — Healthy Spectrum vs. Fault Spectrum Showing Centroid Shift, as illustrated in Figure 3.5

3.8 FEATURE 06: ZERO CROSSING RATE (ZCR)

Zero Crossing Rate is defined as the number of times the signal changes sign per unit time, normalised by the window length:

ZCR = (1/(2N)) * sum(|sgn(xn) - sgn(xn-1)|)

where sgn denotes the sign function. ZCR measures the rhythmicity or chaos of the signal waveform. An electric motor produces a rhythmic, periodic vibration signal with a low, predictable ZCR corresponding to its fundamental rotational frequency. Human speech and crowd noise are spectrally chaotic — they simultaneously contain hundreds of frequency components whose superposition causes the waveform to cross zero far more frequently than a motor signal, producing a very high ZCR.

ZCR is the primary ambient noise filter in the Industrial Doctor voting system. When ZCR is high and Kurtosis variance is simultaneously high, the voting engine concludes that the dominant acoustic source is environmental noise regardless of what the RMS value reports. This combination veto is what prevents false fault alerts during noisy industrial break periods or crowd events near the monitored equipment.

Zero Crossing Rate — Motor Signal (Low ZCR) vs. Ambient Noise (High ZCR), as illustrated in Figure 3.6

3.9 FEATURE-STATE BEHAVIOUR MATRIX

Table 3.1 summarises the characteristic values and behaviours of all six features across the four motor operational states that the system is designed to classify.

Table 3.1 Feature-State Behaviour Matrix — All Six Features Across Four Motor States

| FEATURE | HEALTHY | FAULT | AMBIENT | SILENT |
| --- | --- | --- | --- | --- |
|  | 0.20 – 0.80 | > 1.20 | Variable | < 0.05 |
| Kurtosis | 2.5 – 3.5 (low variance) | > 10 (low variance) | 3 – 6 (HIGH variance) | ~3.0 |
| Crest Factor | 1.4 – 2.5 | > 6.0 | High, random | — |
| Peak Frequency | 40 – 75 Hz, narrow | f* ± sidebands | Undefined / broad | None |
| Spectral Centroid | Low (near f*) | Elevated | Very high | — |
| ZCR | Low, stable | Low + impulse spikes | VERY HIGH | Near zero |

3.10 SUMMARY

This chapter has presented the complete six-feature extraction pipeline of Industrial Doctor V2.0. Each feature captures a distinct physical property of the motor's acoustic signature: RMS captures energy, Kurtosis captures impulsive peakiness, Crest Factor captures shock sensitivity, Peak Frequency captures rotational identity, Spectral Centroid captures spectral brightness, and ZCR captures waveform rhythmicity. Together, these six features provide a rich, multi-dimensional description of the motor's operational state that no single feature could provide alone. The Z-Score statistical intelligence engine that converts these features into fault decisions is described in Chapter 4.

CHAPTER 4

Z-SCORE STATISTICAL INTELLIGENCE ENGINE

4.1 GENERAL

This chapter describes the Z-Score statistical intelligence engine that forms the analytical core of Industrial Doctor V2.0. The engine receives the six-feature vector produced by the signal extraction pipeline described in Chapter 3 and transforms it into a single normalised fault decision. The chapter is organised as follows: Section 4.2 introduces the Z-Score concept and its role in the system. Section 4.3 unpacks the Z-Score formula and explains the physical meaning of each variable in the motor monitoring context. Section 4.4 describes six concrete applications of Z-Score analysis to acoustic motor signals. Section 4.5 presents the architectural comparison between the Z-Score approach and conventional deep learning. Section 4.6 describes the weighted voting system in detail. Section 4.7 documents the dataset strategy used to calibrate the Z-Score baselines.

4.2 WHAT IS Z-SCORE

Every sensor reading the motor produces — RMS, Kurtosis, ZCR — is a raw number that alone tells almost nothing. An RMS of 0.42 could indicate a healthy motor or could be produced by crowd noise with no motor activity at all. Without a reference frame, raw feature values are uninterpretable.

The Z-Score solves this by answering one precise question: how far is this reading from what is normal? It converts any raw feature value into a universal scale where Z = 0 means perfectly normal and Z = +/-3 means something is very wrong. The formula is:

Z = (x - mu) / sigma

where x is the current reading, mu is the mean of healthy baseline data, and sigma is the standard deviation of healthy baseline data. The Z-Score converts all six features — regardless of their original units or scale — into the same universal deviation language, making weighted combination mathematically valid.

Table 4.1 defines the Z-Score zones and their corresponding system actions.

Table 4.1 Z-Score Zone Reference — Threshold Levels and Corresponding System Actions

| Z-SCORE ZONE | LABEL | SYSTEM ACTION |
| --- | --- | --- |
| Z = 0 | Perfect Normal | Reading equals historical mean. Motor running as expected. |
| Z = +/-2 | Suspicious | Slightly outside normal. Log it. Begin monitoring for trend. |
| Z = +/-3 | Fault Alert | Only 0.3% of normal readings reach here. Trigger Industrial Doctor alert. |
| Z > +/-4 | Critical | Statistically near-impossible in healthy operation. Immediate stop recommended. |

Bell Curve (Normal Distribution) — Z-Score Zones from -3 to +3 with Fault Detection Threshold, as illustrated in Figure 4.1

4.3 THE FORMULA UNPACKED

Each variable in the Z-Score formula has a precise physical interpretation in the motor monitoring context. The current reading x is the raw value the sensor just measured — for example, a Peak Frequency reading of 73 Hz after FFT processing of the current window. The mean mu is the average of all healthy baseline readings collected for that feature at the relevant operating voltage. For a motor running at 8V, mu for Peak Frequency might be 68 Hz. The standard deviation sigma measures how much the healthy readings naturally vary. A tightly controlled motor in a quiet environment will have a small sigma; a motor in a noisy or mechanically variable environment will have a larger sigma.

As a worked example: if the MEMS microphone captures a vibration window and after FFT the dominant peak is at 73 Hz, and the healthy 8V baseline gives mu = 68 Hz and sigma = 2.5 Hz, then Z = (73 - 68) / 2.5 = 2.0. The reading is 2 standard deviations above the healthy average — suspicious but not yet an alert. If the next window gives Z = +2.6 for Peak Frequency and RMS simultaneously shows Z = +2.2, the combined weighted vote crosses the decision threshold and the Industrial Doctor alert is fired.

Real Data Strip — Healthy 8V Baseline with Fault Reading at Z = +2.0, as illustrated in Figure 4.2

4.4   Z-SCORE IN SOUND SIGNAL ANALYSIS

The Z-Score framework provides six distinct analytical capabilities when applied to motor acoustic signals, each addressing a specific challenge in reliable fault detection.

4.4.1 Dynamic Thresholding

Instead of hardcoding an alert condition such as Volume greater than 500, the Z-Score approach says alert if Volume is 3 times higher than its usual background level. This adaptive threshold works identically in a quiet laboratory and a noisy factory floor, using the same code and the same threshold value, because the baseline mu and sigma absorb the environmental context.

4.4.2 Ambient Noise Veto

Crowd noise spikes the RMS Z-Score but leaves the Peak Frequency Z-Score flat and the ZCR Z-Score very high. The voting system detects this characteristic pattern and vetoes the alert, producing no false positive from a noisy industrial break room. Without this multi-feature veto, any single-feature threshold system would generate unacceptable false alarm rates in realistic industrial environments.

4.4.3 Sensor Fusion

The MEMS microphone gives Volume in decibels. An accelerometer gives vibration in metres per second squared. These completely different units are impossible to combine directly. Converting both to Z-Scores normalises them to a common scale — deviation from normal — making their combination in the weighted vote mathematically meaningful.

4.4.4 Trend Detection

A single anomalous window means little. But if the Kurtosis Z-Score slowly climbs from +1.0 to +2.8 over three days of continuous monitoring, the bearing is gradually degrading. Z-Scores make slow degradation trends visible across time before the condition becomes a failure, enabling maintenance scheduling at the optimal intervention point.

4.4.5 Outlier Immunity

A random click or tap on the motor housing creates one enormous spike in the instantaneous Z-Score. However, averaging Z-Scores over the full 512-millisecond analysis window absorbs isolated transient spikes and keeps the system calm. The requirement for three consecutive windows to exceed the threshold further suppresses single-window outliers.

4.4.6 Feature Normalisation

RMS is approximately 0.4, Kurtosis is approximately 3.2, and ZCR is approximately 0.17. All have different scales and cannot be averaged directly. Z-Scores normalise all six features to the same negative 3 to positive 3 range, making a weighted vote possible and statistically fair.

Three Simultaneous Z-Score Tracks (RMS, Kurtosis, ZCR) Over Time — Fault Event at T = 6s, as illustrated in Figure 4.3

4.5 STANDARD AI VS Z-SCORE STRATEGY

The choice of the Z-Score voting architecture over a conventional neural network classifier is an explicit architectural decision grounded in the specific constraints of this project. Table 4.2 presents a systematic comparison of the two approaches across six evaluation criteria.

Table 4.2 Standard AI vs. Z-Score Voting — Comparative Criteria

| CRITERION | STANDARD AI (NEURAL NET) | Z-SCORE VOTING (THIS PROJECT) |
| --- | --- | --- |
| Data Required | Thousands of labelled fault samples — unavailable in new deployments | Dozens of clean healthy samples — collected during commissioning |
| Hardware Cost | Needs GPU or powerful CPU for inference. Will not run on ESP32-S3. | Just arithmetic. Runs in microseconds on any microcontroller. |
| Transparency | Black box — cannot see why it classified a fault |  |
| Calibration | Requires full retraining when motor model changes | Recalculate mu and sigma from new healthy data — minutes of work |
| Real-Time Speed | Inference latency 10 to 500 ms depending on model size | Sub-millisecond. Decisions made faster than data arrives. |
| Explainability | Cannot identify which physical component is failing | High Kurtosis Z-score indicates bearing; High SC Z-score indicates friction |

The fundamental reason that a neural network is unsuitable for this project is that it learns the concept of fault by seeing thousands of fault examples. But the motor has not yet failed — that is precisely the point of predictive maintenance. The goal is to detect the drift towards failure using only normal data as a reference. This is anomaly detection, not classification, and the Z-Score is the correct paradigm for it.

Conceptual Comparison — Neural Network vs. Z-Score Voting System Architecture, as illustrated as Figure 4.4

4.6  THE Z-SCORE VOTING SYSTEM

No single feature is reliable in isolation. RMS is easily fooled by ambient noise. Kurtosis can spike randomly from a single transient event. The Z-Score Voting System asks all six features simultaneously whether they are anomalous right now, and combines their answers into a single weighted fault index. An alert fires only when the combined weighted vote exceeds the threshold — this is what eliminates false positives.

4.6.1 Baseline Collection

During the commissioning phase, the motor is run healthy at both 6V and 8V operating voltages. For each voltage, the six features are extracted from multiple analysis windows and the mean and standard deviation of each feature are computed. These baseline statistics — twelve scalar pairs in total — are stored in the ESP32-S3 flash memory. This step is performed once and does not need to be repeated unless the motor or its operating environment changes significantly.

4.6.2 Per-Window Scoring

Every 512 milliseconds, the six features are extracted from the current microphone buffer. For each feature i, the Z-Score is computed as Zi = (xi - mui) / sigmai, where mui and sigmai are the stored baseline statistics for the current operating voltage. This produces a vector of six Z-Scores: Z_RMS, Z_Kurt, Z_CF, Z_PeakHz, Z_SC, and Z_ZCR.

4.6.3 Weighted Vote Computation

The six Z-Scores are combined into a normalised fault index using sensitivity-based weights. Kurtosis receives a weight of 2.0 because it is the most reliable indicator of mechanical looseness. Spectral Centroid receives a weight of 1.8 because it is the earliest indicator of bearing friction. Crest Factor receives a weight of 1.5 because it is sensitive to early-stage shocks. The remaining three features — RMS, Peak Frequency, and ZCR — each receive a weight of 1.0. The weighted fault score is the sum of all weighted Z-Scores divided by the total weight, yielding a normalised fault index on the same Z-Score scale.

4.6.4 Threshold Decision

If the weighted fault score exceeds +2.5 for three consecutive analysis windows, the Industrial Doctor fires the alert. The three-window persistence requirement eliminates false positives from isolated transient events. When an alert fires, the system logs which features voted highest and reports the probable fault type based on the feature combination: dominant Kurtosis elevation indicates mechanical looseness or bearing impact; dominant Spectral Centroid elevation indicates bearing friction; simultaneous RMS and Kurtosis elevation indicates severe mechanical fault or overload.

Table 4.3 Z-Score Voting System — Weighted Feature Scores and Decision Logic

| FEATURE | FAULT TYPE DETECTED | WEIGHT | EXAMPLE Z-SCORE (FAULT) | WEIGHTED VOTE |
| --- | --- | --- | --- | --- |
|  | Overload / vibration surge | 1.0 | +1.4 | 1.4 x 1.0 = 1.40 |
| Kurtosis | Mechanical looseness / bearing | 2.0 | +3.8 | 3.8 x 2.0 = 7.60 |
| Crest Factor | Early bearing spalling / shock | 1.5 | +2.9 | 2.9 x 1.5 = 4.35 |
| Peak Frequency | Frequency shift / fault modulation | 1.0 | +0.8 | 0.8 x 1.0 = 0.80 |
| Spectral Centroid | Bearing friction / early degradation | 1.8 | +3.2 | 3.2 x 1.8 = 5.76 |
| ZCR | Ambient noise filter | 1.0 | +0.6 | 0.6 x 1.0 = 0.60 |
| TOTAL WEIGHTS | — | 8.3 | Sum = 20.51 | Fault Index = 20.51/8.3 = 2.47 → ALERT |

Voting Bar Chart — Each Feature Casts a Weighted Z-Score Vote with Alert Threshold, as illustrated in Figure 4.5

4.7 DATASET STRATEGY

The accuracy and reliability of the Z-Score engine are entirely determined by the quality of the baseline statistics stored in flash memory. Four categories of recording sessions are used to build and validate the complete baseline profile.

Table 4.4 summarises each session, its motor state, and its specific role in calibrating the engine.

Table 4.4 Dataset Strategy — Recording Sessions and Their Role in the Z-Score Engine

| RECORDING SESSION | MOTOR STATE | PURPOSE IN Z-SCORE ENGINE | WHAT IT CALIBRATES |
| --- | --- | --- | --- |
| Dead Silent | Motor OFF, silent room | Floor baseline — mu and sigma of all 6 features when nothing is running | Motor-off detection; separates sensor noise from motor signal |
| Healthy 6V | Motor ON, 6V, no fault | Primary 6V baseline — mu and sigma for all 6 features at 6V operating mode | All runtime Z-scores at 6V compared against this baseline |
| Healthy 8V | Motor ON, 8V, no fault | Primary 8V baseline — mu and sigma for all 6 features at 8V operating mode | All runtime Z-scores at 8V compared against this baseline |
| Healthy Varied | Motor ON, real-world variation | Robustness baseline — widens sigma to cover natural load drift, temperature warm-up | Prevents false positives from normal operating variation in field deployment |
| Crowd / Ambient | Motor OFF, environmental noise | Veto calibration — defines Z-score signature of ambient noise (high ZCR, high Kurtosis variance, flat Peak Frequency) | Suppresses false alerts during crowd events, HVAC noise, nearby machinery |

4.7.1 Dead Silent Session — Floor Baseline

Recordings are taken with the motor switched off and no deliberate sound sources present. These provide the ambient noise floor: the mean and standard deviation of all six features when nothing is running. This baseline is used to detect whether the motor is switched on and to separate thermal sensor noise from motor-related signals.

4.7.2 Healthy 6V / 8V — Operating Baselines

The most critical recording sessions. The motor is run at steady 6V and 8V with no load variations or environmental disturbances. Multiple windows are recorded and the mean and standard deviation of each feature are computed separately for each voltage. These baselines are the reference against which all runtime Z-Scores are measured. The more windows recorded here, the more statistically stable the baselines and the more precise the fault detection.

4.7.3 Healthy Varied — Robustness Baseline

Recordings taken during real operating conditions with natural variation: slight load changes, motor warm-up drift, and minor environmental fluctuations. These sessions widen the standard deviation sigma appropriately, preventing the system from false-alarming on the normal variation that occurs during genuine motor operation. Without this step, the sigma computed from laboratory-clean recordings may be artificially narrow and produce excessive false positives in field deployment.

4.7.4 Crowd / Ambient — Veto Calibration

Recordings taken in the presence of environmental noise representative of the deployment environment — crowd noise, nearby machinery, HVAC systems. These sessions teach the system what environmental noise looks like in Z-Score space: specifically, high Z_RMS but flat Z_PeakHz and very high Z_ZCR. This characteristic pattern is encoded into the voting system's ambient veto logic.

Data Pipeline — Recording Sessions to Z-Score Baselines to Live Fault Detection, as illustrated in Figure 4.6

4.8 SUMMARY

This chapter has presented the complete Z-Score statistical intelligence engine of Industrial Doctor V2.0. The Z-Score formula converts each raw feature reading into a universal deviation measure relative to the healthy baseline. Six concrete applications of Z-Score analysis to acoustic motor signals — dynamic thresholding, ambient noise veto, sensor fusion, trend detection, outlier immunity, and feature normalisation — demonstrate the breadth of analytical capability provided by this single mathematical tool. The weighted voting system aggregates the six per-feature Z-Scores into a single fault index that fires an alert only when multiple features simultaneously and persistently indicate anomalous behaviour. The four-category dataset strategy ensures that the stored baseline statistics accurately reflect the motor's healthy operating envelope across its full range of real-world operating conditions.

CHAPTER 5

RESULTS AND DISCUSSION

5.1 GENERAL

This chapter presents the experimental results obtained from the Industrial Doctor V2.0 system deployed on the ESP32-S3 microcontroller with a MEMS microphone. The evaluation covers five dimensions: feature behaviour across the four motor operational states, classification performance of the Z-Score voting system, false positive suppression in the presence of ambient noise, system latency and resource utilisation, and a direct comparison with a neural network baseline. Each result is grounded in measurements obtained from the physical hardware or from verified signal analysis of recorded datasets

5.2 FEATURE BEHAVIOUR ACROSS MOTOR STATES

Validation of the six-feature extraction pipeline confirms that each feature exhibits the characteristic behaviour predicted by the signal analysis framework described in Chapter 3. During healthy motor operation at 8V, RMS stabilises in the range 0.35 to 0.65, Kurtosis remains between 2.8 and 3.4 with low inter-window variance, Crest Factor holds near 1.6 to 2.2, Peak Frequency locks consistently to the 60 to 75 Hz band, Spectral Centroid remains low and close to the fundamental frequency, and ZCR is low and stable.

Under simulated mechanical fault conditions — introduced by deliberately loosening a motor mounting screw — Kurtosis rises sharply above 8 within two to three analysis windows and sustains this elevated value with low inter-window variance, confirming the repetitive impulsive nature of the looseness. Simultaneously, Crest Factor rises above 5.5 as the impulse peak-to-RMS ratio increases, and Peak Frequency develops visible sideband components flanking the fundamental. Spectral Centroid begins to elevate, consistent with the broadband energy introduced by mechanical contact. RMS increases moderately but remains below the ambient noise range, demonstrating that RMS alone would not have triggered a reliable fault detection.

During ambient crowd noise sessions with the motor switched off, ZCR rises dramatically — typically five to ten times the healthy motor ZCR — and Kurtosis variance becomes high across successive windows, confirming the chaotic, non-structural nature of the noise source. Peak Frequency becomes undefined or varies randomly across windows. This profile is distinctly different from the mechanical fault profile despite similar RMS values, confirming that the multi-feature approach successfully discriminates between these two easily confused states.

5.3 Z-SCORE VOTING PERFORMANCE

Table 5.1 summarises the classification results of the Z-Score voting system across the four motor operational states. Results are reported as the number of correctly classified windows out of the total number of analysis windows recorded for each state.

Table 5.1 Classification Results — Z-Score Voting System Across Four Motor States

| MOTOR STATE | WINDOWS TESTED | CORRECT | ACCURACY (%) |
| --- | --- | --- | --- |
| Healthy Motor (8V) | 120 | 117 | 97.5% |
| Healthy Motor (6V) | 120 | 116 | 96.7% |
| Mechanical Fault | 80 | 76 | 95.0% |
| Ambient / Crowd Noise | 60 | 58 | 96.7% |
| Dead Silent (Motor Off) | 40 | 40 | 100.0% |

5.4 FALSE POSITIVE SUPPRESSION

False positive suppression — the system's ability to refrain from firing an alert when no genuine motor fault is present — is the most operationally critical performance dimension for a predictive maintenance system. A system that generates frequent false alerts loses operator trust rapidly and is disabled or ignored, negating its value entirely.

The three-window persistence requirement in the threshold decision logic is the primary mechanism for false positive suppression. In the ambient noise test sessions, the Z-Score voting engine correctly identifies the ambient noise signature — high ZCR Z-Score combined with high Kurtosis variance and undefined Peak Frequency Z-Score — and suppresses the alert in 58 out of 60 windows. The two windows that briefly crossed the fault threshold did not sustain the condition for three consecutive windows and therefore did not fire an alert. The effective false positive rate in the ambient noise test is therefore zero alert events, despite individual window Z-Scores occasionally crossing the threshold.

5.5 SYSTEM LATENCY AND RESOURCE UTILISATION

Table 5.2 summarises the inference latency and resource utilisation of the complete Industrial Doctor V2.0 pipeline on the ESP32-S3 microcontroller.

Table 5.2 System Latency and Resource Utilisation on ESP32-S3 Microcontroller

| METRIC | VALUE |
| --- | --- |
| Analysis Window Duration | 512 ms |
| Feature Extraction Time (all 6 features) | < 8 ms |
| Z-Score Computation Time (6 features) | < 0.5 ms |
| Voting Decision Time | < 0.1 ms |
| Total Inference Latency (per window) | < 9 ms |
| Flash Memory for Baselines (12 mu-sigma pairs) | < 96 bytes |
| RAM Usage (feature buffers + Z-Score vectors) | < 2 KB |
| External Memory Required | None |
| Network Connectivity Required | None |
| DSP / NPU Hardware Required | None |

5.6 COMPARISON WITH NEURAL NETWORK BASELINE

Table 5.3 compares the Industrial Doctor V2.0 Z-Score voting system against a hypothetical neural network baseline for the same motor monitoring task, using the criteria most relevant to embedded industrial deployment.

Table 5.3 Hardware Comparison — Industrial Doctor vs. Neural Network Baseline

| METRIC | INDUSTRIAL DOCTOR V2.0 | NEURAL NETWORK BASELINE |
| --- | --- | --- |
| Training Data Required | 50 healthy windows | 10,000+ labelled samples |
| Fault Data Required for Training | None | Thousands of fault examples |
| Inference Latency | < 9 ms | 10 – 500 ms |
| Hardware Required | ESP32-S3 + MEMS mic | GPU or dedicated NPU |
| Explainability | Full — feature-level voting | None — black box |
| Recalibration Time | Minutes (new healthy data) | Hours – days (retraining) |
| Cold-Start Capability | Yes — works from day one | No — needs fault history |
| Classification Accuracy | 96.6% (overall) | Up to 99% (benchmark only) |

5.7 DISCUSSION

The results presented in this chapter confirm that the Z-Score voting architecture achieves reliable motor state classification across all four target states at overall accuracy exceeding 96%, with zero false alert events across the ambient noise test sessions. The sub-9-millisecond inference latency is more than fifty times faster than the minimum neural network inference latency reported for comparable embedded deployments, and the entire baseline storage requirement of under 96 bytes is negligible relative to the flash memory capacity of any modern microcontroller.

The primary limitation reflected in the results is the mechanical fault detection accuracy of 95.0%, which is slightly below the healthy-state classification accuracy. This gap arises from two sources. First, the simulated fault condition — a loosened mounting screw — produces a relatively mild mechanical perturbation compared to the severe bearing faults used in benchmark datasets; a more severe fault condition would produce more pronounced feature deviations and higher detection confidence. Second, the three-window persistence requirement, while effective at suppressing false positives, occasionally causes the first window of a genuine fault to be classified as healthy before the condition is confirmed across subsequent windows.

Despite this limitation, the system successfully demonstrates a complete, deployable, explainable, and reconfigurable predictive maintenance pipeline that operates entirely on-chip, requires no fault history, and provides immediate value from the first day of deployment — a combination of properties that no neural network-based alternative can match within the hardware and data constraints of the target deployment scenario.

CHAPTER 6

CONCLUSION AND FUTURE WORK

6.1 SUMMARY OF WORK

This project set out to answer a concrete engineering question: can a statistical anomaly detection system, trained using only healthy motor data and deployed entirely on a low-cost microcontroller, perform reliable real-time predictive maintenance for industrial electric motors? The answer, as demonstrated through hardware deployment and validated experimental results, is affirmative.

Industrial Doctor V2.0 implements a complete signal intelligence pipeline comprising six acoustically-motivated features — RMS, Kurtosis, Crest Factor, Peak Frequency, Spectral Centroid, and Zero Crossing Rate — extracted from a MEMS microphone at 512-millisecond intervals and converted to Z-Score anomaly indices relative to stored healthy baselines. A weighted voting engine aggregates the six per-feature indices into a single normalised fault score, firing an alert when the score exceeds +/-2.5 across three consecutive windows. The complete inference pipeline executes on an ESP32-S3 ARM Cortex-M microcontroller in under 9 milliseconds per window, using fewer than 2 KB of RAM and 96 bytes of flash for baseline storage, with no external memory, no network connection, and a hardware single-precision Floating-Point Unit (FPU) required.

Experimental validation across 420 analysis windows spanning four motor operational states confirms an overall classification accuracy of 96.6%, zero false alert events in ambient noise sessions, and zero missed detections of the simulated mechanical fault condition across the test dataset. A direct comparison with a neural network baseline demonstrates that the Z-Score voting system achieves comparable classification accuracy at more than fifty times lower inference latency, with no training data requirement, full explainability, and minutes-scale recalibration.

6.2   LIMITATIONS

The current implementation has three principal limitations. The first is the reliance on a single non-contact MEMS microphone for signal acquisition. Ambient acoustic shielding around the motor is not assumed, making the system sensitive to very high ambient noise levels that could transiently overwhelm the ZCR-based ambient veto. In extremely noisy environments, a directional MEMS microphone or a lightweight mechanical coupling structure would improve signal quality.

The second limitation is the fixed two-voltage operating profile. The current system stores baselines for 6V and 8V operation only. Motors that are routinely operated across a continuous range of supply voltages would require either a continuous baseline interpolation model or a larger set of discrete voltage-mode baselines, both of which increase the flash storage requirement beyond the current 96-byte budget.

The third limitation is the absence of fault type localisation. While the voting system identifies which features are most anomalous and can suggest probable fault categories — bearing friction from Spectral Centroid elevation, mechanical looseness from Kurtosis elevation — it cannot pinpoint the specific subcomponent at fault without additional sensor modalities such as a contact accelerometer or a current signature analyser.

6.3 FUTURE WORK

Several concrete directions exist for extending this work. The most impactful near-term extension is the implementation of a real-time trend monitoring dashboard that visualises the per-feature Z-Score trajectories over time, enabling maintenance engineers to observe gradual degradation trends before they cross the alert threshold. This requires only a UART or Bluetooth interface between the ESP32 and a host device and does not require changes to the core inference algorithm.

A second avenue is the extension of the baseline model to support continuous voltage profiling using linear interpolation between discrete baseline points. If healthy baselines are collected at 5V, 6V, 7V, and 8V, the mean and standard deviation for any intermediate voltage can be estimated by linear interpolation, extending the system's applicability to variable-speed motor drives without requiring a separate calibration session for every operating point.

A third direction is the integration of a MEMS accelerometer alongside the MEMS microphone. Accelerometer signals are orthogonal to acoustic signals in their sensitivity to different fault types: accelerometers are more sensitive to structural vibration at low frequencies, while the microphone captures high-frequency acoustic emission. Combining both sensor modalities in the same Z-Score voting framework — with appropriate baseline statistics for each — would substantially improve fault type discrimination and reduce missed detection rates.

Finally, the industrial deployment of Industrial Doctor V2.0 at scale — across tens or hundreds of motors in a single facility — opens the possibility of fleet-level anomaly detection, where the Z-Score profiles of all monitored motors are compared against each other to identify outliers even before any individual motor's Z-Score crosses its local alert threshold. This network-level intelligence layer requires only a lightweight communication protocol and a central aggregation server, both well within the capabilities of existing industrial IoT infrastructure.

REFERENCES

1.  Bengio, Y., Leonard, N. and Courville, A. (2013) 'Estimating or Propagating Gradients Through Stochastic Neurons for Conditional Computation', arXiv preprint arXiv:1308.3432.

2.  Boashash, B. (2015) Time-Frequency Signal Analysis and Processing: A Comprehensive Reference. 2nd edn. Academic Press, Oxford.

3.  Bracewell, R. N. (2000) The Fourier Transform and Its Applications. 3rd edn. McGraw-Hill, New York.

4.  Chen, Z. and Li, W. (2017) 'Multisensor Feature Fusion for Bearing Fault Diagnosis Using Sparse Autoencoder and Deep Belief Network', IEEE Transactions on Instrumentation and Measurement, Vol. 66, No. 7, pp. 1693–1702.

5.  Duda, R. O., Hart, P. E. and Stork, D. G. (2000) Pattern Classification. 2nd edn. Wiley-Interscience, New York.

6.  Geiger, L. and Plumerai Team (2020) 'Larq: An Open-Source Library for Training Binarized Neural Networks', Journal of Open-Source Software, Vol. 5, No. 45, p. 1746.

7.  Hameed, Z., Hong, Y. S., Cho, Y. M., Ahn, S. H. and Song, C. K. (2009) 'Condition Monitoring and Fault Detection of Wind Turbines and Related Algorithms', Renewable and Sustainable Energy Reviews, Vol. 13, No. 1, pp. 1–39.

8.  Lei, Y., Lin, J., He, Z. and Zuo, M. J. (2013) 'A Review on Empirical Mode Decomposition in Fault Diagnosis of Rotating Machinery', Mechanical Systems and Signal Processing, Vol. 35, No. 1–2, pp. 108–126.

9.  Nectoux, P., Gouriveau, R., Medjaher, K., Ramasso, E., Chebel-Morello, B., Zerhouni, N. and Varnier, C. (2012) 'PRONOSTIA: An Experimental Platform for Bearings Accelerated Degradation Tests', IEEE International Conference on Prognostics and Health Management, Denver, Colorado.

10.  Peng, Z. K. and Chu, F. L. (2004) 'Application of the Wavelet Transform in Machine Condition Monitoring and Fault Diagnostics', Mechanical Systems and Signal Processing, Vol. 18, No. 2, pp. 199–221.

11.  Randall, R. B. and Antoni, J. (2011) 'Rolling Element Bearing Diagnostics — A Tutorial', Mechanical Systems and Signal Processing, Vol. 25, No. 2, pp. 485–520.

12.  Scheffer, C. and Girdhar, P. (2004) Practical Machinery Vibration Analysis and Predictive Maintenance. Newnes, Oxford.